EKF Series Beveling Machines Technical Comparison

This document provides a comprehensive technical breakdown and side-by-side comparison of the EKF series beveling machines across models 300, 450, 452, 530, 545, 630, and 645 based on official technical datasheets.

Key Differences Across Model Classes

Chamfer Capabilities: Models 300, 530, and 630 feature a fixed 30° chamfer angle with maximum width capacities scaling from 5 mm up to 12 mm and 15 mm. Models 450, 545, and 645 offer a fixed 45° angle with matching widths (6 mm, 12 mm, and 15 mm). Model EKF 452 is a specialized radius milling variant strictly built for a Radius 2.5 mm finish.

Motor Power & Speed Control: The EKF 300 series utilizes a 1500 W motor running at a fixed 6000 rpm. The EKF 500 series steps up to an 1800 W motor with variable speed adjustments (1850–3350 rpm), while the heavy-duty EKF 600 series maximizes power at 2300 W with an adjustable 1600–3000 rpm range.

Milling Tool Setup: Tooling scalability correlates directly with class sizes. The 300 class uses 3 carbide inserts, upgrading to 6 carbide inserts in the 500 class, and maximizing performance with 10 carbide inserts in the 600 class.

Complete Technical Specification Table

| Technical Data | EKF 300 | EKF 450 | EKF 452 | EKF 530 | EKF 545 | EKF 630 | EKF 645 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chamfer Angle | 30° | 45° | Radius 2.5 mm | 30° | 45° | 30° | 45° |
| Chamfer Width | 0–5 mm | 0–6 mm | Radius 2.5 mm | 0–12 mm | 0–12 mm | 0–15 mm | 0–15 mm |
| Cutter Type | ZFR 300.3 | ZFR 450.3 | ZFR 452.3 | ZFR 530 | ZFR 545 | ZFR 630 | ZFR 645 |
| Carbide Inserts | 3 inserts | 3 inserts | 3 inserts | 6 inserts | 6 inserts | 10 inserts | 10 inserts |
| Dimensions (LxBxH) | 310 × 295 × 138 mm | 310 × 295 × 138 mm | 310 × 295 × 138 mm | 375 × 350 × 300 mm | 375 × 350 × 300 mm | 400 × 350 × 300 mm | 400 × 350 × 300 mm |
| Approx. Net Weight | 4.6 kg | 4.6 kg | 4.6 kg | 9.3 kg | 9.3 kg | 10.2 kg | 10.2 kg |
| Motor Output | 1500 W | 1500 W | 1500 W | 1800 W | 1800 W | 2300 W | 2300 W |
| Speed (rpm) | 6000 fixed | 6000 fixed | 6000 fixed | 1850–3350 var. | 1850–3350 var. | 1600–3000 var. | 1600–3000 var. |
| Feed Type | Manual | Manual | Manual | Manual | Manual | Manual | Manual |
| Protection Type | IP 20 | IP 20 | IP 20 | IP 20 | IP 20 | IP 20 | IP 20 |

Shared Electrical & Environmental Baseline

All evaluated EKF models feature dual-voltage configuration compatibility (operating efficiently at 220–240 V / 50–60 Hz as well as 110–125 V / 50–60 Hz). Additionally, the entire product family depends on Manual feeding mechanisms and provides an IP 20 standard international protection classification safety framework.